BAXZ 1313

Data Communications and Networking

Question Bank

Quick-Fire Q&A for Rapid Final-Exam Review

How to use this file

Cover the answer, attempt each question, then check. All numeric examples differ from the Golden Summary and Mock Exam so you practice the METHOD, not memorized numbers.

Contents

1. Foundations & Protocols

2. Signals & Channel Capacity

3. Analog Transmission

4. Digital Transmission

5. Multiplexing

6. Error Detection & Correction

7. Flow Control & ARQ

8. IPv4 & VLSM

9. IPv6

# SECTION 1: FOUNDATIONS & PROTOCOLS

Q: List the 5 PDU names from Application layer down to Physical layer.

A: Data → Segment → Packet → Frame → Bits.

Q: What are the 5 components of data communication?

A: Message, Sender, Receiver, Medium, Protocol.

Q: Name the 3 key elements of a protocol.

A: Syntax (structure/format), Semantics (meaning of bits), Timing (when/how fast data is sent).

Q: What are the 4 fundamental characteristics required for effective data communication?

A: Delivery, Accuracy, Timeliness, Jitter.

Q: Differentiate simplex, half-duplex, and full-duplex transmission modes.

A: Simplex: one direction only. Half-duplex: both directions, but not simultaneously. Full-duplex: both directions at the same time.

Q: Which OSI layer is responsible for logical addressing and routing?

A: The Network layer.

Q: Which OSI layer handles framing, MAC addressing, and error detection between adjacent nodes?

A: The Data Link layer.

# SECTION 2: SIGNALS & CHANNEL CAPACITY

Q: Define analog and digital signals in terms of possible values.

A: Analog: infinite number of values, continuous. Digital: finite number of values, usually just 0 and 1.

Q: A signal has a period of 0.5 ms. Find its frequency.

Formula: f = 1 / T

f = 1 / 0.0005 = 2000 Hz = 2 kHz

Q: A sine wave is s(t) = 20 sin(2π·2000t + π/4). Find amplitude, frequency, and phase in degrees.

A = 20

f = 2000 Hz

φ = (π/4) × (180/π) = 45°

Q: A signal is shifted by 1/8 of a cycle. Find the phase shift in degrees and radians.

Degrees: 360° / 8 = 45°

Radians: 45° × (π/180°) = π/4 rad

Q: A composite signal has components at 150, 350, 550, 750 Hz. Find the bandwidth.

BW = f_highest − f_lowest

BW = 750 − 150 = 600 Hz

Q: A baseband signal spans 100–900 Hz. Can it pass through a medium that only passes 3000–4000 Hz? Why?

A: No. The medium's passband does not overlap the signal's frequency range at all, so the entire signal is filtered out / lost.

Q: A digital link has a bit rate of 8 Mbps. Find the bit interval in nanoseconds.

Bit interval = 1 / Bit rate = 1 / 8,000,000 = 0.000000125 s

= 125 ns

Q: A noisy channel has B = 3 MHz and SNR = 255. Find the Shannon capacity, then the number of signal levels (L) needed via Nyquist.

Shannon: C = B log₂(1+SNR) = 3,000,000 × log₂(256) = 3,000,000 × 8 = 24,000,000 bps = 24 Mbps

Nyquist: C = 2BL₂(L) → 24,000,000 = 2 × 3,000,000 × log₂(L)

log₂(L) = 4 → L = 16

Q: If solving for L gives a non-power-of-2 value (e.g. 11.3), what should a network engineer do?

A: Round UP to the next power of 2 (e.g. choose L = 16), since digital hardware cannot implement fractional signal levels and rounding down would fail to support the required capacity.

Q: Name the three main transmission impairments.

A: Attenuation (loss of signal strength), Distortion (change in signal shape), Noise (external interference).

# SECTION 3: ANALOG TRANSMISSION

Q: Define bit rate vs. baud rate.

A: Bit rate = bits transmitted per second. Baud rate = signal units (symbols) transmitted per second. Baud rate ≤ bit rate; equal only when 1 bit per signal unit.

Q: A signal carries 4 bits per signal unit at 4800 baud. Find the bit rate.

Bit Rate = Baud Rate × bits per signal unit

= 4800 × 4 = 19,200 bps

Q: How does ASK encode binary data, and what is its main weakness?

A: Binary 1 = full amplitude carrier, binary 0 = zero/low amplitude. Weakness: highly susceptible to noise, since noise easily distorts amplitude.

Q: How many bits per signal unit does 8-PSK carry?

A: log₂(8) = 3 bits per signal unit.

Q: A full-duplex ASK system has a total bandwidth from 1000 Hz to 9000 Hz. Find the bandwidth per direction and each carrier frequency.

Bandwidth per direction = (9000−1000)/2 = 4000 Hz

Forward: 1000–5000 Hz, carrier = (1000+5000)/2 = 3000 Hz

Backward: 5000–9000 Hz, carrier = (5000+9000)/2 = 7000 Hz

Q: A 64-QAM system transmits at 3000 baud. Find the bit rate.

bits per signal unit = log₂(64) = 6

Bit Rate = 3000 × 6 = 18,000 bps

Q: An audio signal has bandwidth 4 kHz. Find the AM and FM bandwidths (broadcast approximation).

BW(AM) = 2 × 4 = 8 kHz

BW(FM) ≈ 10 × 4 = 40 kHz

Q: Why does FM need more bandwidth than AM?

A: FM produces multiple sidebands due to frequency deviation/modulation index, spreading energy over a wider range; AM only produces two simple sidebands proportional to the message bandwidth.

# SECTION 4: DIGITAL TRANSMISSION

Q: What are the two key characteristics line coding must provide?

A: Synchronization (clock recovery) and DC balance (no DC component).

Q: A digital system uses 8 signal levels at a pulse rate of 2000 pulses/sec. Find the bit rate.

bits per signal unit = log₂(8) = 3

Bit Rate = Pulse Rate × log₂(L) = 2000 × 3 = 6000 bps

Q: Compare NRZ-L and Manchester encoding for synchronization.

A: NRZ-L has no built-in synchronization mechanism and can develop a DC component during long bit runs. Manchester guarantees a transition every bit interval, enabling self-synchronization and removing the DC component, at the cost of double the bandwidth.

Q: In Differential Manchester encoding, what determines the bit value?

A: Not the mid-bit transition (which always occurs for sync) — the bit value is determined by the presence or absence of an extra transition at the START of the bit interval.

Q: Why is a DC component undesirable in digital transmission?

A: It cannot pass through transformers/capacitors, wastes power, and causes baseline wander at the receiver, leading to decoding errors.

Q: An analog signal spans 2000–18,000 Hz and is converted from band-pass to low-pass before sampling. Find the minimum sampling rate.

When band-pass → low-pass conversion happens first, sampling rate = 2 × Bandwidth

Bandwidth = 18,000 − 2,000 = 16,000 Hz

Sampling rate = 2 × 16,000 = 32,000 samples/sec

Q: What is aliasing and when does it occur?

A: Aliasing occurs when the sampling rate is below the Nyquist rate (2 × f_max); different frequencies become indistinguishable, producing a false reconstructed frequency.

Q: A signal needs 20 quantization levels, sampled at 6000 samples/sec. Find bits per sample and bit rate.

n = ⌈log₂(20)⌉ = ⌈4.32⌉ = 5 bits (2⁴=16 not enough, 2⁵=32 sufficient)

Bit Rate = 6000 × 5 = 30,000 bps

# SECTION 5: MULTIPLEXING

Q: What is the difference between FDM and TDM?

A: FDM (analog): divides bandwidth into separate frequency bands, all signals sent simultaneously. TDM (digital): all signals share the same frequency but take turns in time slots.

Q: What is a guard band and why is it needed?

A: A narrow, unused buffer range of frequencies placed between adjacent channels in FDM, to prevent crosstalk / interference from one channel leaking into a neighboring channel.

Q: Differentiate Synchronous TDM and Statistical TDM.

A: Synchronous TDM: fixed pre-assigned time slots per source, even if idle (can waste slots). Statistical TDM: slots assigned dynamically only to sources with data, more efficient.

Q: What is WDM typically used for?

A: Combining multiple optical signals of different wavelengths onto a single fiber-optic cable — conceptually FDM for light.

# SECTION 6: ERROR DETECTION & CORRECTION

Q: Differentiate single-bit error and burst error.

A: Single-bit error: only one bit is corrupted. Burst error: 2+ consecutive bits are corrupted; burst length = distance between first and last corrupted bit.

Q: Why are burst errors more common than single-bit errors?

A: Noise/interference duration usually exceeds the time to transmit one bit, so it corrupts multiple consecutive bits at high bit rates.

Q: Why do data-link protocols prefer error detection over error correction?

A: Correction needs far more redundant bits and processing overhead; since retransmission is fast in modern networks, detection + retransmission (e.g. CRC) is more efficient.

Q: Calculate the even parity bit for the data unit 1100101, then write the final transmitted data.

A: Count of 1s in 1100101 = 4 (even) → parity bit = 0 → transmitted: 11001010.

Q: What does 2D (block) parity improve on compared to simple 1D parity?

A: It computes parity for both rows and columns, catching many burst/multi-bit error patterns that 1D parity alone would miss (e.g. an even number of errors in one row may still be caught via column parity).

Q: State the principle of CRC.

A: Treats the data as a polynomial; the sender performs modulo-2 (XOR) division by a fixed divisor and appends the remainder (CRC) to the data; the receiver re-divides the full codeword — remainder 0 means no error detected.

Q: Given dataword = 1011001 and divisor = 1011, find the CRC remainder and the final codeword.

A: Append 3 zeros (divisor length − 1): 1011001000. Perform modulo-2 (XOR) division by 1011.

Remainder (CRC) = 011

Final codeword = 1011001 + 011 = 1011001011

Q: Name two advantages of CRC over simple parity check.

A: 1) Detects a much wider range of errors, including most burst errors. 2) Provides strong protection with relatively few added redundant bits (good overhead-to-protection ratio).

# SECTION 7: FLOW CONTROL & ARQ

Q: What problem does flow control solve?

A: Prevents a fast sender from overwhelming a slow receiver's buffer by limiting how much unacknowledged data can be in transit.

Q: Describe Stop-and-Wait ARQ and its main drawback.

A: Sender sends one frame and waits for an ACK before sending the next. Drawback: inefficient, especially over long-delay links, since the link sits idle while waiting.

Q: In Go-Back-N ARQ, what happens when one frame in the window is lost?

A: The receiver discards that frame and all subsequent frames already received (even if correct); the sender must retransmit the lost frame and every frame sent after it.

Q: State ONE primary advantage of Selective-Repeat ARQ over Go-Back-N.

A: Higher bandwidth efficiency: Selective-Repeat retransmits only the specific corrupted/lost frame, and buffers correctly-received out-of-order frames instead of discarding them — eliminating Go-Back-N's wasteful bulk retransmissions.

Q: What is the trade-off Selective-Repeat makes to achieve this efficiency?

A: It requires more complex receiver logic and additional buffer memory to store and reorder out-of-sequence frames.

# SECTION 8: IPv4 & VLSM

Q: What is the formula for usable host addresses in a subnet?

A: Usable hosts = 2ⁿ − 2, where n = number of host bits (subtract network address and broadcast address).

Q: What CIDR prefix is needed for a subnet requiring 100 usable hosts?

A: Need 2ⁿ−2 ≥ 100 → n=7 gives 126 hosts → CIDR = /25 (32−7).

Q: List the VLSM allocation procedure in order.

A: 1) Sort subnets largest→smallest by host count. 2) For each, find smallest block size (2ⁿ) where 2ⁿ−2 ≥ required hosts. 3) Allocate largest subnet first from the base address. 4) Next subnet starts at (previous network address + previous block size). 5) Repeat for remaining subnets.

Q: Using base 172.16.0.0/24, design VLSM subnets for Sales (50 users), IT (25 users), HR (10 users). Show the full allocation table.

A: Sort largest→smallest: Sales (50) → IT (25) → HR (10).

| Dept | Hosts | CIDR | Network | First Valid | Last Valid | B'cast |
| --- | --- | --- | --- | --- | --- | --- |
| Sales | 50 | /26 | 172.16.0.0 | 172.16.0.1 | 172.16.0.62 | .63 |
| IT | 25 | /27 | 172.16.0.64 | 172.16.0.65 | 172.16.0.94 | .95 |
| HR | 10 | /28 | 172.16.0.96 | 172.16.0.97 | 172.16.0.110 | .111 |

Q: How many addresses are wasted for the Sales subnet above?

A: Block size 64 → 62 usable hosts; used = 50; wasted = 62 − 50 = 12 addresses.

# SECTION 9: IPv6

Q: How many bits and groups make up an IPv6 address?

A: 128 bits, written as 8 groups of 4 hexadecimal digits each.

Q: Expand the shortened IPv6 address 2001:DB8::1:0:0:1 into full unabbreviated form.

A: Visible groups: 2001, DB8, 1, 0, 0, 1 = 6 groups, so :: represents 2 missing zero groups.

2001:0DB8:0000:0000:0001:0000:0000:0001

Q: What is the rule for using the double-colon (::) shorthand?

A: It can replace one or more consecutive all-zero groups, but can only be used ONCE in a given address.

Q: Compare IPv4 and IPv6 header length.

A: IPv4: variable, 20–60 bytes. IPv6: fixed, always 40 bytes.

Q: List the three basic IPv6 address types and their delivery pattern.

A: Unicast (one-to-one), Multicast (one-to-many, prefix FF00::/8), Anycast (one-to-nearest). IPv6 has no broadcast — replaced by multicast.

Q: Name the four IPv6 transition mechanisms.

A: Dual stack, Tunneling, NAT64, Header translation.

Q: State two advantages of IPv6 over IPv4 related to security or header structure.

A: 1) Native/mandatory IPsec integration. 2) Simplified fixed-length header format (also acceptable: no router-level fragmentation).